ОТЧЕТ ПО ЛАБОРАТОРНОЙ РАБОТЕ 5

Тема: Подключение проводной и беспроводной локальных сетей в Cisco Packet Tracer

# Цель работы

Собрать топологию проводной и беспроводной сети в Cisco Packet Tracer, подобрать правильные типы кабелей для разных соединений, проверить связность между узлами, а также изучить физическое представление сети в программе.

# Основные задачи лабораторной работы

1) Подключить Cloud к Router0 и к Cable Modem.

2) Подключить Router0 к Router1, серверу netacad.pka и к терминалу настройки.

3) Подключить Router1 к Switch, а Cable Modem к Wireless Router и Family PC.

4) Проверить доступность узлов с помощью ping и Web Browser.

5) Перейти в Physical Workspace и ответить на вопросы по локациям.

# Исходные данные и адресация

Адресация взята из задания. Для удобства я выписал ключевые IP-адреса.

IP-адрес Router0 F0/0: 192.168.2.1/24

IP-адрес Router0 F0/1: 10.0.0.1/24

IP-адрес Router0 Ser0/0/0: 172.31.0.1/24

IP-адрес Router1 Ser0/0: 172.31.0.2/24

IP-адрес Router1 F1/0: 172.16.0.1/24

IP-адрес Switch F0/1: 172.16.0.2

IP-адрес сервера netacad.pka: 10.0.0.254

IP-адрес Wireless Router (LAN): 192.168.1.1

# Ход выполнения работы

## Часть 1. Подключение к облаку

Через “Connections” с иконкой оранжевой молнии подключил Cloud к Router0 и к Cable Modem. Для Cloud Eth6 и Router0 F0/0 использован Copper Straight-Through. Для Cloud Coax7 и Cable Modem Port0 использован коаксиальный кабель.

Рисунок: Логическая топология после подключения устройств

Вывод: если кабель выбран правильно, индикаторы линков становятся зелеными.

## Часть 2. Подключение маршрутизатора Router0

Router0 и Router1 соединил Serial кабелем по Ser0/0/0 и Ser0/0. Router0 подключил к серверу netacad.pka по F0/1, для этого использовал медный кросс-кабель. Для доступа к CLI выполнил консольное подключение Router0 Console к RS232 на Configuration Terminal.

Вывод: консольный кабель не дает сетевого доступа, он нужен для входа в CLI и проверки интерфейсов.

## Часть 3. Подключение оставшихся устройств

Подключил Router1 к Switch. Затем Cable Modem подключил к Wireless Router через порт Internet. Wireless Router подключил кабелем к Family PC. Для этих соединений использовался Copper Straight-Through.

Вывод: после подключения иногда нужно подождать несколько секунд, пока интерфейсы станут активными.

## Часть 4. Проверка подключений

Проверил доступность сервера netacad.pka по ping и через Web Browser. Затем проверил ping с Home PC до Switch и проверил интерфейсы Router0 командой show ip interface brief.

Задание 4.1. Ping Family PC до netacad.pka. Я смотрю на строку статистики, где видно “Lost = 0”.

Pinging 10.0.0.254 with 32 bytes of data:

Reply from 10.0.0.254: bytes=32 time=2ms TTL=126

Reply from 10.0.0.254: bytes=32 time=4ms TTL=126

Reply from 10.0.0.254: bytes=32 time=6ms TTL=126

Reply from 10.0.0.254: bytes=32 time=4ms TTL=126

Ping statistics for 10.0.0.254:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 2ms, Maximum = 6ms, Average = 4ms

Задание 4.1. Открытие “http://netacad.pka” в браузере.

Рисунок: Web Browser, страница netacad.pka

Задание 4.2. Ping Home PC до Switch. Красным выделены таймауты, зеленым выделено “Lost = 0”.

Home -> switch

Cisco Packet Tracer PC Command Line 1.0

C:\>ping 172.16.0.2

Pinging 172.16.0.2 with 32 bytes of data:

Request timed out.

Request timed out.

Reply from 172.16.0.2: bytes=32 time=14ms TTL=252

Reply from 172.16.0.2: bytes=32 time=21ms TTL=252

Ping statistics for 172.16.0.2:

Packets: Sent = 4, Received = 2, Lost = 2 (50% loss),

Approximate round trip times in milli-seconds:

Minimum = 14ms, Maximum = 21ms, Average = 17ms

C:\>ping 172.16.0.2

Pinging 172.16.0.2 with 32 bytes of data:

Reply from 172.16.0.2: bytes=32 time=10ms TTL=252

Reply from 172.16.0.2: bytes=32 time=11ms TTL=252

Reply from 172.16.0.2: bytes=32 time=13ms TTL=252

Reply from 172.16.0.2: bytes=32 time=15ms TTL=252

Ping statistics for 172.16.0.2:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 10ms, Maximum = 15ms, Average = 12ms

C:\>

Задание 4.3. Проверка Router0. Важно увидеть, что интерфейсы в работе имеют состояние up up.

Configuration terminal -> terminal

Router0>enable

Router0#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

FastEthernet0/0        192.168.2.1     YES manual up                    up

FastEthernet0/1        10.0.0.1        YES manual up                    up

Serial0/0/0            172.31.0.1      YES manual up                    up

Serial0/0/1            unassigned      YES unset  administratively down down

Vlan1                  unassigned      YES unset  administratively down down

Router0#

Вывод по части 4: связность подтверждена. Потери на первом ping обычно связаны с ARP, повторный ping идет стабильнее.

## Часть 5. Изучение физической топологии

Перешел во вкладку Physical и посмотрел локации Cloud, Primary Network, Secondary Network и Home Network. Ниже ответы на вопросы из задания.

Шаг 1с. Cloud. В синей стойке подключено два провода: к Router0 и к Cable Modem.

Рисунок: Cloud в Physical

Шаг 2а. Primary Network. В таблице справа отображаются элементы локации и соединения. Это помогает понять, какие порты соединены между собой.

Рисунок: Primary Network в Physical

Рисунок: Таблица в Primary Network

Шаг 3а. Secondary Network. По два оранжевых кабеля на устройство сделаны потому, что один кабель внутри стойки, а второй уходит в кабельный канал на другую локацию. Так проще обслуживать и раскладывать кабели.

Рисунок: Secondary Network в Physical

Шаг 4а. Домашняя сеть накрыта овальной сеткой, потому что это зона покрытия Wi-Fi.

Шаг 4б. Стойки нет, потому что это домашняя среда и оборудование размещается на мебели (столы в нашем случае).

Рисунок: Home Network в Physical

# Дополнительное исследование

Для повышения оценки я выбрал исследование “Конфигурация DHCP-сервера”. Идея такая: устройства домашней сети получают IP-адрес автоматически, и настройка клиента занимает пару кликов.

## Настройка DHCP и проверка

В GUI Wireless Router включил DHCP. Адрес роутера 192.168.1.1, пул выдачи начинается с 192.168.1.100, чтобы адрес роутера не пересекался с выдаваемыми адресами.

Рисунок: Wireless Router, параметры DHCP

На Home PC выбрал DHCP. На скрине видно “DHCP request successful”, а также полученные параметры: IP 192.168.1.100, шлюз 192.168.1.1 и DNS 10.0.0.254.

Рисунок: Home PC, IP Configuration

Проверка ping после настройки DHCP. Я выделил результаты, на которые опираюсь: отсутствие потерь до 192.168.1.1 и стабильный ping до 10.0.0.254 на втором запуске.

Создаём VLAN на Switch

Press RETURN to get started!

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

%LINK-3-UPDOWN: Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to down

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

Switch>enable

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#vlan 10

Switch(config-vlan)#name VLAN10

Switch(config-vlan)#vlan 20

Switch(config-vlan)#name VLAN20

Switch(config-vlan)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Gig1/1, Gig2/1, Gig3/1

Gig4/1, Gig5/1, Gig6/1, Gig7/1

Gig8/1, Gig9/1

10   VLAN10                           active

20   VLAN20                           active

1002 fddi-default                     active

1003 token-ring-default               active

1004 fddinet-default                  active

1005 trnet-default                    active

Switch#

trunk на порту к Router1 (Fa0/1)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#interface fa0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

show interfaces trunk

Port        Mode         Encapsulation  Status        Native vlan

Fa0/1       on           802.1q         trunking      1

Port        Vlans allowed on trunk

Fa0/1       1-1005

Port        Vlans allowed and active in management domain

Fa0/1       1,10,20

Port        Vlans in spanning tree forwarding state and not pruned

Fa0/1       none

Switch#show interfaces status

Port      Name               Status       Vlan       Duplex  Speed Type

Fa0/1                        connected    trunk      a-full  a-100 10/100BaseTX

Gig1/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig2/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig3/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig4/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig5/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig6/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig7/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig8/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig9/1                       notconnect   1          auto    auto  10/100/1000BaseTX

C:\>ping 192.168.1.1

Pinging 192.168.1.1 with 32 bytes of data:

Reply from 192.168.1.1: bytes=32 time=20ms TTL=255

Reply from 192.168.1.1: bytes=32 time=10ms TTL=255

Reply from 192.168.1.1: bytes=32 time=8ms TTL=255

Reply from 192.168.1.1: bytes=32 time=15ms TTL=255

Ping statistics for 192.168.1.1:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 8ms, Maximum = 20ms, Average = 13ms

C:\>ping 10.0.0.254

Pinging 10.0.0.254 with 32 bytes of data:

Request timed out.

Reply from 10.0.0.254: bytes=32 time=10ms TTL=126

Reply from 10.0.0.254: bytes=32 time=16ms TTL=126

Reply from 10.0.0.254: bytes=32 time=11ms TTL=126

Ping statistics for 10.0.0.254:

Packets: Sent = 4, Received = 3, Lost = 1 (25% loss),

Approximate round trip times in milli-seconds:

Minimum = 10ms, Maximum = 16ms, Average = 12ms

C:\>ping 10.0.0.254

Pinging 10.0.0.254 with 32 bytes of data:

Reply from 10.0.0.254: bytes=32 time=11ms TTL=126

Reply from 10.0.0.254: bytes=32 time=9ms TTL=126

Reply from 10.0.0.254: bytes=32 time=14ms TTL=126

Reply from 10.0.0.254: bytes=32 time=11ms TTL=126

Ping statistics for 10.0.0.254:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 9ms, Maximum = 14ms, Average = 11ms

Вывод по доп. исследованию: DHCP работает. Home PC получил параметры сети автоматически и имеет связность до шлюза и сервера.

## Пояснение по логам VLAN и trunk

Это не отдельное доп. задание. Я оставил как пояснение, где именно в выводах show смотреть, что искали и что нашли. В логе подсвечены ключевые слова.

Что искал: VLAN10 и VLAN20 со статусом active. Дальше искал строку, где Fa0/1 trunking и используется 802.1Q. Затем искал, что активны VLAN 1,10,20. И отдельно искал строку про spanning-tree forwarding state и значение none. Также смотрел notconnect на гигабитных портах.

Создаём VLAN на Switch

Switch>enable

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#vlan 10

Switch(config-vlan)#name VLAN10

Switch(config-vlan)#vlan 20

Switch(config-vlan)#name VLAN20

Switch(config-vlan)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/1, Gig1/1, Gig2/1, Gig3/1

Gig4/1, Gig5/1, Gig6/1, Gig7/1

Gig8/1, Gig9/1

10   VLAN10                           active

20   VLAN20                           active

1002 fddi-default                     active

1003 token-ring-default               active

1004 fddinet-default                  active

1005 trnet-default                    active

Switch#

trunk на порту к Router1 (Fa0/1)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#interface fa0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

show interfaces trunk

Port        Mode         Encapsulation  Status        Native vlan

Fa0/1       on           802.1q         trunking      1

Port        Vlans allowed on trunk

Fa0/1       1-1005

Port        Vlans allowed and active in management domain

Fa0/1       1,10,20

Port        Vlans in spanning tree forwarding state and not pruned

Fa0/1       none

Switch#show interfaces status

Port      Name               Status       Vlan       Duplex  Speed Type

Fa0/1                        connected    trunk      a-full  a-100 10/100BaseTX

Gig1/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig2/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig3/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig4/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig5/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig6/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig7/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig8/1                       notconnect   1          auto    auto  10/100/1000BaseTX

Gig9/1                       notconnect   1          auto    auto  10/100/1000BaseTX

C:\>ping 192.168.1.1

Pinging 192.168.1.1 with 32 bytes of data:

Reply from 192.168.1.1: bytes=32 time=20ms TTL=255

Reply from 192.168.1.1: bytes=32 time=10ms TTL=255

Reply from 192.168.1.1: bytes=32 time=8ms TTL=255

Reply from 192.168.1.1: bytes=32 time=15ms TTL=255

Ping statistics for 192.168.1.1:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 8ms, Maximum = 20ms, Average = 13ms

C:\>ping 10.0.0.254

Pinging 10.0.0.254 with 32 bytes of data:

Request timed out.

Reply from 10.0.0.254: bytes=32 time=10ms TTL=126

Reply from 10.0.0.254: bytes=32 time=16ms TTL=126

Reply from 10.0.0.254: bytes=32 time=11ms TTL=126

Ping statistics for 10.0.0.254:

Packets: Sent = 4, Received = 3, Lost = 1 (25% loss),

Approximate round trip times in milli-seconds:

Minimum = 10ms, Maximum = 16ms, Average = 12ms

C:\>ping 10.0.0.254

Pinging 10.0.0.254 with 32 bytes of data:

Reply from 10.0.0.254: bytes=32 time=11ms TTL=126

Reply from 10.0.0.254: bytes=32 time=9ms TTL=126

Reply from 10.0.0.254: bytes=32 time=14ms TTL=126

Reply from 10.0.0.254: bytes=32 time=11ms TTL=126

Ping statistics for 10.0.0.254:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 9ms, Maximum = 14ms, Average = 11ms

Где это видно: VLAN10 и VLAN20 отображаются как active. В show interfaces trunk видно, что Fa0/1 trunking и используется 802.1Q. В “Vlans allowed and active in management domain” указаны VLAN 1,10,20. Значение none появляется потому, что нет активных портов access с конечными устройствами, а гигабитные порты имеют notconnect.

# Общий вывод

Лабораторная работа 5 выполнена полностью. Топология собрана, кабели выбраны правильно, связность подтверждена ping и открытием web-страницы сервера. В Physical Workspace изучено физическое представление сети и даны ответы на вопросы.

Дополнительное исследование по DHCP выполнено и проверено на практике. Логи оформлены с подсветкой ключевых строк, чтобы было видно, где именно подтверждается вывод.