Лабораторная работа № 4

Тема: “Packet Tracer. Настройка начальных параметров коммутатора”.

Среда: Cisco Packet Tracer.

Цель работы

Освоить базовую настройку коммутатора Cisco, обеспечить защищенный доступ к CLI через консоль, настроить пароли для привилегированного режима, включить шифрование паролей, настроить баннер MOTD и сохранить конфигурацию в NVRAM.

Основные задачи лабораторной работы

1) Проверить конфигурацию коммутатора по умолчанию.

2) Настроить имя устройства и безопасный доступ к консоли и привилегированному режиму.

3) Настроить баннер MOTD.

4) Сохранить конфигурацию в NVRAM и проверить startup-config.

5) Повторить настройку на втором коммутаторе S2.

Ход работы

Часть 1. Проверка конфигурации коммутатора по умолчанию

В этой части выполнен вход в привилегированный режим EXEC и просмотр текущей конфигурации, чтобы определить состав интерфейсов и параметры линий удаленного доступа.

Команды и вывод:

Switch>enable
Switch#show running-config
Building configuration...

Current configuration : 1080 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
no service password-encryption
!
hostname Switch
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
!
!
!
line con 0
!
line vty 0 4
 login
line vty 5 15
 login
!
!
!
!
end

Ответы на контрольные вопросы по результатам show running-config:

1) Интерфейсов FastEthernet: 24.

2) Интерфейсов GigabitEthernet: 2.

3) Диапазон VTY-линий: 0-15 (разделено на 0-4 и 5-15).

4) Команда для просмотра содержимого NVRAM: show startup-config.

5) Сообщение “startup-config is not present” появляется, когда конфигурация еще не была сохранена в NVRAM и находится только в оперативной памяти.

Вывод по части 1: конфигурация по умолчанию содержит интерфейсы FastEthernet и GigabitEthernet, а также линии VTY для удаленного доступа, но пароли и сохраненный startup-config отсутствуют.

Часть 2. Настройка основных параметров коммутатора S1

Выполнено присвоение имени устройству, настройка пароля на консоль, настройка пароля enable password, установка enable secret, а также шифрование паролей.

Последовательность действий и подтверждающие логи:

Switch>enable
Switch#show running-config
Building configuration...

Current configuration : 1080 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
no service password-encryption
!
hostname Switch
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
!
!
!
line con 0
!
line vty 0 4
 login
line vty 5 15
 login
!
!
!
!
end



Switch#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
Switch(config)#hostname S1
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console



Switch#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
Switch(config)#hostname S1
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console

S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#line console 0
S1(config-line)#password letmein
S1(config-line)#login
S1(config-line)#exit
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console
exit









S1 con0 is now available






Press RETURN to get started.













User Access Verification

Password: 

S1>^Z
S1>enable
S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#enable password c1$c0
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console
exit









S1 con0 is now available






Press RETURN to get started.













User Access Verification

Password: 

S1>enable
Password: 
S1#show running-config
Building configuration...

Current configuration : 1125 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
no service password-encryption
!
hostname S1
!
enable password c1$c0
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
!
!
!
line con 0
 password letmein
 login
!
line vty 0 4
 login
line vty 5 15
 login
!
!
!
!
end


S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#enable secret itsasecret
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console

S1#show running-config
Building configuration...

Current configuration : 1172 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
no service password-encryption
!
hostname S1
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password c1$c0
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
!
!
!
line con 0
 password letmein
 login
!
line vty 0 4
 login
line vty 5 15
 login
!
!
!
!
end



S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#service password-encryption
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console
show run
Building configuration...

Current configuration : 1189 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S1
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
 --More-- 
 


S1>enable
Password: 
S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#banner motd "This is a secure system. Authorized Access Only!"
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console

copy running-config startup-config 
Destination filename [startup-config]? 
Building configuration...
[OK]
S1#copy run start
Destination filename [startup-config]? 
Building configuration...
[OK]
S1#show startup-config 
Using 1252 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S1
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
banner motd ^CThis is a secure system. Authorized Access Only!^C
!
!
!
line con 0
 password 7 082D495A041C0C19
 login
!
line vty 0 4
 login
line vty 5 15
 login
 --More--

Пояснение по командам:

hostname S1: задает имя коммутатора, чтобы приглашение CLI было однозначным.

line console 0, password letmein, login: включают аутентификацию по паролю при доступе через консоль.

enable password c1$c0: задает пароль привилегированного режима, но он виден в конфиге открыто до включения шифрования.

enable secret itsasecret: задает защищенный пароль привилегированного режима в виде хэша, он имеет приоритет над enable password.

service password-encryption: включает шифрование текстовых паролей (тип 7) в конфигурации.

Вывод по части 2: доступ к консоли и привилегированному режиму защищен паролями, а после включения service password-encryption пароли в конфигурации отображаются в зашифрованном виде.

Часть 3. Настройка баннера MOTD

Настроен баннер MOTD, который отображается всем пользователям при входе и предупреждает о запрете несанкционированного доступа.

Команда настройки:

S1(config)#banner motd "This is a secure system. Authorized Access Only!"
S1(config)#exit
S1#

Ответы на вопросы:

1) Баннер отображается при подключении к коммутатору перед вводом учетных данных (при входе через консоль и VTY).

2) Баннер MOTD нужен как предупреждение о запрещенном доступе и как краткое сообщение для сетевого персонала.

Вывод по части 3: баннер настроен и выполняет роль предупреждения для всех пользователей, открывающих CLI.

Часть 4. Сохранение файлов конфигурации в NVRAM

После настройки выполнено сохранение running-config в startup-config, чтобы конфигурация не терялась при перезапуске.

Команды и вывод:

copy running-config startup-config 
Destination filename [startup-config]? 
Building configuration...
[OK]
S1#copy run start
Destination filename [startup-config]? 
Building configuration...
[OK]
S1#show startup-config 
Using 1252 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S1
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
banner motd ^CThis is a secure system. Authorized Access Only!^C
!
!
!
line con 0
 password 7 082D495A041C0C19
 login
!
line vty 0 4
 login
line vty 5 15
 login

Самая короткая версия команды copy running-config startup-config: copy run start.

Вывод по части 4: конфигурация успешно сохранена, и startup-config соответствует настроенному running-config.

Часть 5. Настройка коммутатора S2

Во второй части работы настройки повторены на S2: hostname, пароли, баннер, шифрование паролей и сохранение конфигурации.

Логи настройки S2:

Switch>enable
Switch#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
Switch(config)#
Switch(config)#hostname S2
S2(config)#
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console
configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#
S2(config)#line console 0
S2(config-line)#
S2(config-line)#password letmein
S2(config-line)#login
S2(config-line)#exit
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#enable password c1$c0
S2(config)#enable secret itsasecret
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#banner motd "Authorized access only. Unauthorized access is prohibited and violators will be prosecuted to the full extent of the law."
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#service password-encryption
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#show run 
Building configuration...

Current configuration : 1325 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S2
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
 --More-- 
 
 
S2#exit









S2 con0 is now available






Press RETURN to get started.











Authorized access only. Unauthorized access is prohibited and violators will be prosecuted to the full extent of the law.

User Access Verification

Password: 

User Access Verification

Password: 

S2>show run
        ^
% Invalid input detected at '^' marker.
	
S2>enable
Password: 
S2#show run
Building configuration...

Current configuration : 1325 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S2
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
 --More--

Вывод по части 5: S2 настроен аналогично S1, баннер отображается при входе, пароли зашифрованы и конфигурация сохранена.

Дополнительное исследование

Эксперимент: VLAN 10 и trunk между S1 и S2

Цель эксперимента: создать VLAN 10, назначить порты ПК в access VLAN 10, настроить trunk на межкоммутаторном соединении и проверить связность между ПК.

Задачи эксперимента:

1) Создать VLAN 10 на S1 и S2.

2) Назначить access-порты ПК в VLAN 10.

3) Включить trunk на линке S1-S2 и разрешить VLAN 10.

4) Проверить VLAN и trunk командами show vlan brief и show interfaces trunk.

5) Настроить IP-адреса на PC1 и PC2 и выполнить ping.

Логи настройки на S1:

S1>enable
Password: 
Password: 
S1#enable
S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#vlan 10
S1(config-vlan)#name USERS
S1(config-vlan)#exit
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console'


S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#interface fastEthernet0/1
S1(config-if)#switchport mode access
S1(config-if)#switchport access vlan 10
S1(config-if)#exit
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console


S1#show vlan brief

VLAN Name                             Status    Ports
---- -------------------------------- --------- -------------------------------
1    default                          active    Fa0/2, Fa0/3, Fa0/4, Fa0/5
                                                Fa0/6, Fa0/7, Fa0/8, Fa0/9
                                                Fa0/10, Fa0/11, Fa0/12, Fa0/13
                                                Fa0/14, Fa0/15, Fa0/16, Fa0/17
                                                Fa0/18, Fa0/19, Fa0/20, Fa0/21
                                                Fa0/22, Fa0/23, Fa0/24, Gig0/1
                                                Gig0/2
10   USERS                            active    Fa0/1
1002 fddi-default                     active    
1003 token-ring-default               active    
1004 fddinet-default                  active    
1005 trnet-default                    active    



S1#show interfaces trunk
Port        Mode         Encapsulation  Status        Native vlan
Fa0/1       on           802.1q         trunking      1

Port        Vlans allowed on trunk
Fa0/1       10

Port        Vlans allowed and active in management domain
Fa0/1       10

Port        Vlans in spanning tree forwarding state and not pruned
Fa0/1       none

S1#
%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (1), with S2 FastEthernet0/1 (10).

S1#conf t
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#interface fa0/2
S1(config-if)#switchport mode access
S1(config-if)#switchport access vlan 10
S1(config-if)#end
S1#
%SYS-5-CONFIG_I: Configured from console by console

Логи настройки на S2:

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#vlan 10
S2(config-vlan)#name USERS
S2(config-vlan)#exit
S2(config)#interface fastEthernet0/1
S2(config-if)#switchport mode access
S2(config-if)#switchport access vlan 10
S2(config-if)#exit
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#interface gigabitEthernet0/1
S2(config-if)#switchport mode trunk
S2(config-if)#switchport trunk allowed vlan 10
S2(config-if)#
S2(config-if)#exit
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#
%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (10), with S1 FastEthernet0/1 (1).
%SPANTREE-2-RECV_PVID_ERR: Received 802.1Q BPDU on non trunk FastEthernet0/1 VLAN1.

%SPANTREE-2-BLOCK_PVID_LOCAL: Blocking FastEthernet0/1 on VLAN0001. Inconsistent port type.


S2#conf t
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#interface fa0/2
S2(config-if)#switchport mode access
S2(config-if)#switchport access vlan 10
S2(config-if)#end
S2#
%SYS-5-CONFIG_I: Configured from console by console

%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (10), with S1 FastEthernet0/1 (1).

S2#interface fa0/1
             ^
% Invalid input detected at '^' marker.
	
S2#conf t
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#interface fa0/1
S2(config-if)#switchport mode trunk

S2(config-if)#
%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

S2(config-if)#switchport trunk allowed vlan 10
S2(config-if)#end
S2#
%SYS-5-CONFIG_I: Configured from console by console

Проверка связности между ПК:

C:\>ping 192.168.10.3

Pinging 192.168.10.3 with 32 bytes of data:

Reply from 192.168.10.3: bytes=32 time<1ms TTL=128
Reply from 192.168.10.3: bytes=32 time<1ms TTL=128
Reply from 192.168.10.3: bytes=32 time<1ms TTL=128
Reply from 192.168.10.3: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.10.3:
    Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),
Approximate round trip times in milli-seconds:
    Minimum = 0ms, Maximum = 0ms, Average = 0ms

C:\>

Краткий разбор результата:

После создания VLAN 10 и назначения access-портов трафик внутри VLAN проходит только при корректно настроенном trunk между коммутаторами.

В процессе появилась типовая проблема “Native VLAN mismatch” и сообщения STP о несогласованном типе порта. Это произошло из-за различий настроек порта между S1 и S2. После приведения обоих портов к trunk и ограничения allowed VLAN до 10 несоответствие устраняется, а VLAN 10 проходит через линк.

Вывод по эксперименту: VLAN 10 создан, trunk настроен, ping между ПК успешен, что подтверждает передачу трафика VLAN 10 через межкоммутаторный линк.

Вопросы безопасности по дополнительному заданию

1) VLAN выполняет сегментацию уровня L2, но не является полной защитой. Для изоляции трафика между VLAN на уровне L3 нужны ACL или межсетевой экран.

2) При неправильной настройке trunk возможны атаки типа VLAN hopping, поэтому следует ограничивать список разрешенных VLAN и не оставлять trunk в состоянии “разрешено все”.

3) Пользовательские порты рекомендуется держать строго в режиме access и при необходимости применять Port Security.

4) Несоответствие Native VLAN и ошибки STP показывают, что ошибки конфигурации могут приводить к блокировке порта и потере связи, поэтому проверки show vlan brief и show interfaces trunk обязательны.

Общий вывод по лабораторной работе

В ходе работы выполнена базовая настройка коммутаторов S1 и S2: задано имя, настроена защита консольного доступа и привилегированного режима, включено шифрование паролей, настроены предупреждающие баннеры MOTD и выполнено сохранение конфигурации в NVRAM.

Дополнительное исследование показало, как VLAN и trunk используются для логической сегментации сети, и почему согласованность настроек trunk на обеих сторонах линка критична для работоспособности сети.

Приложение А. Полные логи основной части (S1 и S2)

Switch>enable
Switch#show running-config
Building configuration...

Current configuration : 1080 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
no service password-encryption
!
hostname Switch
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
!
!
!
line con 0
!
line vty 0 4
 login
line vty 5 15
 login
!
!
!
!
end



Switch#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
Switch(config)#hostname S1
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console



Switch#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
Switch(config)#hostname S1
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console

S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#line console 0
S1(config-line)#password letmein
S1(config-line)#login
S1(config-line)#exit
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console
exit









S1 con0 is now available






Press RETURN to get started.













User Access Verification

Password: 

S1>^Z
S1>enable
S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#enable password c1$c0
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console
exit









S1 con0 is now available






Press RETURN to get started.













User Access Verification

Password: 

S1>enable
Password: 
S1#show running-config
Building configuration...

Current configuration : 1125 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
no service password-encryption
!
hostname S1
!
enable password c1$c0
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
!
!
!
line con 0
 password letmein
 login
!
line vty 0 4
 login
line vty 5 15
 login
!
!
!
!
end


S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#enable secret itsasecret
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console

S1#show running-config
Building configuration...

Current configuration : 1172 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
no service password-encryption
!
hostname S1
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password c1$c0
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
!
!
!
line con 0
 password letmein
 login
!
line vty 0 4
 login
line vty 5 15
 login
!
!
!
!
end



S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#service password-encryption
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console
show run
Building configuration...

Current configuration : 1189 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S1
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
 --More-- 
 


S1>enable
Password: 
S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#banner motd "This is a secure system. Authorized Access Only!"
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console

copy running-config startup-config 
Destination filename [startup-config]? 
Building configuration...
[OK]
S1#copy run start
Destination filename [startup-config]? 
Building configuration...
[OK]
S1#show startup-config 
Using 1252 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S1
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
interface FastEthernet0/1
!
interface FastEthernet0/2
!
interface FastEthernet0/3
!
interface FastEthernet0/4
!
interface FastEthernet0/5
!
interface FastEthernet0/6
!
interface FastEthernet0/7
!
interface FastEthernet0/8
!
interface FastEthernet0/9
!
interface FastEthernet0/10
!
interface FastEthernet0/11
!
interface FastEthernet0/12
!
interface FastEthernet0/13
!
interface FastEthernet0/14
!
interface FastEthernet0/15
!
interface FastEthernet0/16
!
interface FastEthernet0/17
!
interface FastEthernet0/18
!
interface FastEthernet0/19
!
interface FastEthernet0/20
!
interface FastEthernet0/21
!
interface FastEthernet0/22
!
interface FastEthernet0/23
!
interface FastEthernet0/24
!
interface GigabitEthernet0/1
!
interface GigabitEthernet0/2
!
interface Vlan1
 no ip address
 shutdown
!
banner motd ^CThis is a secure system. Authorized Access Only!^C
!
!
!
line con 0
 password 7 082D495A041C0C19
 login
!
line vty 0 4
 login
line vty 5 15
 login
 --More-- 
 
 




Switch2

Switch>enable
Switch#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
Switch(config)#
Switch(config)#hostname S2
S2(config)#
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console
configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#
S2(config)#line console 0
S2(config-line)#
S2(config-line)#password letmein
S2(config-line)#login
S2(config-line)#exit
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#enable password c1$c0
S2(config)#enable secret itsasecret
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#banner motd "Authorized access only. Unauthorized access is prohibited and violators will be prosecuted to the full extent of the law."
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#service password-encryption
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#show run 
Building configuration...

Current configuration : 1325 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S2
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
 --More-- 
 
 
S2#exit









S2 con0 is now available






Press RETURN to get started.











Authorized access only. Unauthorized access is prohibited and violators will be prosecuted to the full extent of the law.

User Access Verification

Password: 

User Access Verification

Password: 

S2>show run
        ^
% Invalid input detected at '^' marker.
	
S2>enable
Password: 
S2#show run
Building configuration...

Current configuration : 1325 bytes
!
version 12.2
no service timestamps log datetime msec
no service timestamps debug datetime msec
service password-encryption
!
hostname S2
!
enable secret 5 $1$mERr$ILwq/b7kc.7X/ejA4Aosn0
enable password 7 08221D0A0A49
!
!
!
!
!
!
spanning-tree mode pvst
spanning-tree extend system-id
!
 --More--

Приложение Б. Полные логи дополнительного задания VLAN 10

S1>enable
Password: 
Password: 
S1#enable
S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#vlan 10
S1(config-vlan)#name USERS
S1(config-vlan)#exit
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console'


S1#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#interface fastEthernet0/1
S1(config-if)#switchport mode access
S1(config-if)#switchport access vlan 10
S1(config-if)#exit
S1(config)#exit
S1#
%SYS-5-CONFIG_I: Configured from console by console


S1#show vlan brief

VLAN Name                             Status    Ports
---- -------------------------------- --------- -------------------------------
1    default                          active    Fa0/2, Fa0/3, Fa0/4, Fa0/5
                                                Fa0/6, Fa0/7, Fa0/8, Fa0/9
                                                Fa0/10, Fa0/11, Fa0/12, Fa0/13
                                                Fa0/14, Fa0/15, Fa0/16, Fa0/17
                                                Fa0/18, Fa0/19, Fa0/20, Fa0/21
                                                Fa0/22, Fa0/23, Fa0/24, Gig0/1
                                                Gig0/2
10   USERS                            active    Fa0/1
1002 fddi-default                     active    
1003 token-ring-default               active    
1004 fddinet-default                  active    
1005 trnet-default                    active    



S1#show interfaces trunk
Port        Mode         Encapsulation  Status        Native vlan
Fa0/1       on           802.1q         trunking      1

Port        Vlans allowed on trunk
Fa0/1       10

Port        Vlans allowed and active in management domain
Fa0/1       10

Port        Vlans in spanning tree forwarding state and not pruned
Fa0/1       none

S1#
%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (1), with S2 FastEthernet0/1 (10).

S1#conf t
Enter configuration commands, one per line.  End with CNTL/Z.
S1(config)#interface fa0/2
S1(config-if)#switchport mode access
S1(config-if)#switchport access vlan 10
S1(config-if)#end
S1#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#vlan 10
S2(config-vlan)#name USERS
S2(config-vlan)#exit
S2(config)#interface fastEthernet0/1
S2(config-if)#switchport mode access
S2(config-if)#switchport access vlan 10
S2(config-if)#exit
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#configure terminal
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#interface gigabitEthernet0/1
S2(config-if)#switchport mode trunk
S2(config-if)#switchport trunk allowed vlan 10
S2(config-if)#
S2(config-if)#exit
S2(config)#exit
S2#
%SYS-5-CONFIG_I: Configured from console by console

S2#
%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (10), with S1 FastEthernet0/1 (1).
%SPANTREE-2-RECV_PVID_ERR: Received 802.1Q BPDU on non trunk FastEthernet0/1 VLAN1.

%SPANTREE-2-BLOCK_PVID_LOCAL: Blocking FastEthernet0/1 on VLAN0001. Inconsistent port type.


S2#conf t
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#interface fa0/2
S2(config-if)#switchport mode access
S2(config-if)#switchport access vlan 10
S2(config-if)#end
S2#
%SYS-5-CONFIG_I: Configured from console by console

%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (10), with S1 FastEthernet0/1 (1).

S2#interface fa0/1
             ^
% Invalid input detected at '^' marker.
	
S2#conf t
Enter configuration commands, one per line.  End with CNTL/Z.
S2(config)#interface fa0/1
S2(config-if)#switchport mode trunk

S2(config-if)#
%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

S2(config-if)#switchport trunk allowed vlan 10
S2(config-if)#end
S2#
%SYS-5-CONFIG_I: Configured from console by console

C:\>ping 192.168.10.3

Pinging 192.168.10.3 with 32 bytes of data:

Reply from 192.168.10.3: bytes=32 time<1ms TTL=128
Reply from 192.168.10.3: bytes=32 time<1ms TTL=128
Reply from 192.168.10.3: bytes=32 time<1ms TTL=128
Reply from 192.168.10.3: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.10.3:
    Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),
Approximate round trip times in milli-seconds:
    Minimum = 0ms, Maximum = 0ms, Average = 0ms

C:\>