Отчет по лабораторной работе №3 по дисциплине “Клиент-серверные приложения и сетевые технологии”

# Цель работы

Изучить физические характеристики межсетевых устройств в Cisco Packet Tracer, научиться подбирать модули расширения для требуемых интерфейсов и корректно соединять устройства. Дополнительно освоить базовый консольный доступ к оборудованию Cisco через терминальную программу и выполнить простую настройку времени.

# Основные задачи лабораторной работы

Определить порты управления маршрутизатора и состав LAN и WAN интерфейсов.

Определить пропускную способность интерфейсов по умолчанию и количество слотов расширения.

Выбрать модули расширения для подключения ПК и оптоволоконного гигабитного канала, установить их и проверить появление интерфейсов.

Соединить устройства в топологии нужными типами кабелей согласно таблице соединений и получить максимальный балл Packet Tracer.

Получить доступ к коммутатору по консольному кабелю, выполнить команды просмотра параметров и вручную настроить часы.

# Выполнение лабораторной работы 3.1 “Изучение межсетевых устройств”

## Часть 1. Определение физических характеристик межсетевых устройств

Шаг 1. Определение портов управления маршрутизатора East.

На задней панели маршрутизатора East доступны порты управления: Console, AUX и USB (mini-USB).

Шаг 2. Определение LAN и WAN интерфейсов маршрутизатора East и проверка через CLI.

LAN интерфейсы: 2 порта GigabitEthernet (0/0 и 0/1). WAN интерфейсы: 4 последовательных Serial (0/0/0, 0/0/1, 0/1/0, 0/1/1).

Вывод show ip interface brief до установки дополнительных модулей:

East>show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0 unassigned YES NVRAM administratively down down GigabitEthernet0/1 unassigned YES NVRAM administratively down down Serial0/0/0 unassigned YES unset down down Serial0/0/1 unassigned YES unset down down Serial0/1/0 unassigned YES unset down down Serial0/1/1 unassigned YES unset down down Vlan1 unassigned YES NVRAM administratively down down

Физические интерфейсы: 6 (2 GigabitEthernet и 4 Serial). Интерфейс Vlan1 виртуальный, его не учитываем.

Шаг 2c. Определение пропускной способности по умолчанию.

Интерфейс GigabitEthernet0/0:

East>show interface gigabitethernet 0/0 GigabitEthernet0/0 is administratively down, line protocol is down (disabled) MTU 1500 bytes, BW 100000 Kbit, DLY 100 usec, Full-duplex, 100Mb/s, media type is RJ45

По умолчанию BW для GigabitEthernet0/0: 100000 Kbit.

Интерфейс Serial0/0/0:

East>show interface serial 0/0/0 Serial0/0/0 is down, line protocol is down (disabled) Hardware is HD64570 MTU 1500 bytes, BW 1544 Kbit, DLY 20000 usec, Encapsulation HDLC, loopback not set, keepalive set (10 sec)

По умолчанию BW для Serial0/0/0: 1544 Kbit.

Шаг 3. Слоты расширения.

Маршрутизатор East: 2 слота eHWIC. Коммутатор Switch2: 6 слотов расширения (module bays).

## Часть 2. Выбор подходящих модулей для подключения и их установка

Шаг 1. Подбор модулей.

Для подключения трех ПК (PC1, PC2, PC3) к маршрутизатору без отдельного коммутатора выбран модуль HWIC-4ESW. Он дает 4 switching-порта, то есть можно подключить до 4 узлов.

Для организации оптоволоконного гигабитного подключения между Switch2 и Switch3 выбран модуль PT-SWITCH-NM-1FGE, который добавляет гигабитный оптический интерфейс (GBIC/SFP слот).

Шаг 2. Установка модулей и проверка появления интерфейсов.

После установки HWIC-4ESW на маршрутизаторе East появились интерфейсы FastEthernet0/1/0-0/1/3:

East>show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0 unassigned YES NVRAM administratively down down GigabitEthernet0/1 unassigned YES NVRAM administratively down down Serial0/0/0 unassigned YES unset down down Serial0/0/1 unassigned YES unset down down FastEthernet0/1/0 unassigned YES unset down down FastEthernet0/1/1 unassigned YES unset down down FastEthernet0/1/2 unassigned YES unset down down FastEthernet0/1/3 unassigned YES unset down down Vlan1 unassigned YES NVRAM administratively down down

По формату интерфейсов FastEthernet0/1/x видно, что модуль установлен в слот 0/1.

На маршрутизаторе West установлен модуль с последовательными интерфейсами:

West>show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0 unassigned YES NVRAM administratively down down GigabitEthernet0/1 unassigned YES NVRAM administratively down down Serial0/0/0 unassigned YES NVRAM administratively down down Serial0/0/1 unassigned YES NVRAM administratively down down Vlan1 unassigned YES NVRAM administratively down down

## Часть 3. Подключение устройств в топологии

Соединения выполнены согласно таблице соединений в задании. Использованы типы кабелей: Copper Straight-Through, Copper Cross-Over, Fiber и Serial DCE (сначала к маршрутизатору East).

После исправления одной ошибки соединения (East GigabitEthernet0/1 подключен к Switch4 GigabitEthernet0/1 прямым медным кабелем) итоговый балл Packet Tracer доведен до 52/52.

Дополнительная проверка после включения интерфейсов (no shutdown) и поднятия линии Serial:

East#show controllers serial 0/0/0 Interface Serial0/0/0 Hardware is PowerQUICC MPC860 DCE V.35, clock rate 2000000

East#show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0 unassigned YES NVRAM up up GigabitEthernet0/1 unassigned YES NVRAM up up Serial0/0/0 unassigned YES unset up up Serial0/0/1 unassigned YES unset down down FastEthernet0/1/0 unassigned YES unset up up FastEthernet0/1/1 unassigned YES unset up up FastEthernet0/1/2 unassigned YES unset up up FastEthernet0/1/3 unassigned YES unset down down Vlan1 unassigned YES NVRAM administratively down down

West#show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0 unassigned YES NVRAM administratively down down GigabitEthernet0/1 unassigned YES NVRAM administratively down down Serial0/0/0 unassigned YES NVRAM up up Serial0/0/1 unassigned YES NVRAM administratively down down Vlan1 unassigned YES NVRAM administratively down down

Оптоволоконный линк между Switch3 и Switch2 после включения интерфейсов GigabitEthernet5/1 также поднялся (up/up).

Switch#show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet5/1 unassigned YES manual up up

## Промежуточный вывод по ЛР 3.1

В ходе работы удалось определить типы интерфейсов и порты управления, подобрать необходимые модули расширения и корректно собрать топологию. Ключевой вывод: в Packet Tracer важно подключать устройства строго по указанным интерфейсам и типам кабелей, иначе автоматическая проверка снижает оценку.

# Выполнение лабораторной работы 3.2 “Запуск сеанса консоли с помощью программы Tera Term”

Работа выполнялась в среде Packet Tracer, где вместо Tera Term использовался встроенный терминал ПК (Desktop, Terminal).

## Часть 1. Доступ к коммутатору через консольный порт

ПК PC1 подключен к консольному порту коммутатора Switch1 консольным кабелем (Console): PC1 RS232 - Switch1 Console. Далее открыт Desktop, Terminal на PC1 с параметрами порта по умолчанию 9600 8N1 без управления потоком.

## Часть 2. Отображение и настройка основных параметров устройства

После установления сеанса выполнена команда show version для определения версии IOS и модели коммутатора.

Switch1>show version Cisco IOS Software, C2960 Software (C2960-LANBASE-M), Version 12.2(25)FX, RELEASE SOFTWARE (fc1) ROM: C2960 Boot Loader (C2960-HBOOT-M) Version 12.2(25r)FX, RELEASE SOFTWARE (fc4) Cisco WS-C2960-24TT processor (revision C0) with 21039K bytes of memory. 24 FastEthernet/IEEE 802.3 interface(s) 2 GigabitEthernet/IEEE 802.3 interface(s)

Версия IOS по выводу: 12.2(25)FX. Модель коммутатора: WS-C2960-24TT.

Далее проверено текущее время командой show clock, затем выполнена настройка времени через enable и clock set. После повторного show clock время изменилось на заданное значение.

## Вопросы для повторения

1. Как предотвратить несанкционированный доступ к устройству Cisco через консольный порт:

Минимальный набор мер: задать пароль на line console 0 и включить login, настроить enable secret, ограничить физический доступ к оборудованию (шкаф, помещение).

2. Достоинства и недостатки последовательной консоли по сравнению с USB консолью:

Последовательная консоль (RJ-45) более универсальна и совместима с разным оборудованием, но часто требует USB-COM адаптер. USB консоль удобнее на ноутбуках, но может требовать драйверы, а также нельзя одновременно использовать USB и RJ-45, потому что USB имеет приоритет.

## Промежуточный вывод по ЛР 3.2

Встроенный терминал Packet Tracer подходит для отработки навыков консольного доступа. При зависании на сообщении Translating ... удобно прерывать выполнение комбинацией Ctrl+Shift+6, затем X, и заранее отключать domain lookup.

# Дополнительное задание: конфигурация коммутатора с VLAN

В качестве дополнительного исследования выбран вариант “Конфигурация коммутатора”. Цель: настроить VLAN, назначить порты, настроить trunk и проверить связность внутри VLAN.

## Краткое описание выполненных действий

Созданы VLAN 10, 20, 30 на коммутаторах и заданы имена VLAN10_USERS, VLAN20_USERS, VLAN30_USERS.

Порты PC4-PC6 на Switch1 назначены в VLAN10, порты PC7-PC9 на Switch2 назначены в VLAN20.

Межкоммутаторные соединения настроены как trunk с разрешенными VLAN 10,20,30.

Настроены SVI для VLAN10 на Switch1-Switch4, назначены IP адреса управления 192.168.10.2-192.168.10.5.

Для проверки связи назначены IP адреса на ПК и выполнены ping внутри VLAN10 и VLAN20, потерь нет.

## Логи конфигурации и проверки (приложение)

Ниже приведены основные команды и выводы из процесса настройки VLAN. Блоки логов оформлены моноширинным шрифтом с серой подложкой.

Создаём VLAN на каждом свитче Switch1> Switch1>enable Switch1#configure terminal Enter configuration commands, one per line. End with CNTL/Z. Switch1(config)# Switch1(config)#vlan 10 Switch1(config-vlan)#name VLAN10_USERS Switch1(config-vlan)#exit Switch1(config)#vlan 20 Switch1(config-vlan)#name VLAN20_USERS Switch1(config-vlan)#exit Switch1(config)#vlan 30 Switch1(config-vlan)#name VLAN30_USERS Switch1(config-vlan)#exit Switch1(config)#end Switch1# %SYS-5-CONFIG_I: Configured from console by console write memory Building configuration... [OK] Switch1# Switch>enable Switch#configure terminal Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#vlan 10 Switch(config-vlan)#name VLAN10_USERS Switch(config-vlan)#exit Switch(config)#vlan 20 Switch(config-vlan)#name VLAN20_USERS Switch(config-vlan)#exit Switch(config)#vlan 30 Switch(config-vlan)#name VLAN30_USERS Switch(config-vlan)#exit Switch(config)#end Switch# %SYS-5-CONFIG_I: Configured from console by console write memory Building configuration... [OK] Switch# Switch1 Настроить access-порты под ПК Switch1#enable Switch1#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch1(config)#int range fa0/1 - 3 Switch1(config-if-range)#switchport mode access Switch1(config-if-range)#switchport access vlan 10 Switch1(config-if-range)#spanning-tree portfast %Warning: portfast should only be enabled on ports connected to a single host. Connecting hubs, concentrators, switches, bridges, etc... to this interface when portfast is enabled, can cause temporary bridging loops. Use with CAUTION %Portfast has been configured on FastEthernet0/1 but will only have effect when the interface is in a non-trunking mode. %Warning: portfast should only be enabled on ports connected to a single host. Connecting hubs, concentrators, switches, bridges, etc... to this interface when portfast is enabled, can cause temporary bridging loops. Use with CAUTION %Portfast has been configured on FastEthernet0/2 but will only have effect when the interface is in a non-trunking mode. %Warning: portfast should only be enabled on ports connected to a single host. Connecting hubs, concentrators, switches, bridges, etc... to this interface when portfast is enabled, can cause temporary bridging loops. Use with CAUTION %Portfast has been configured on FastEthernet0/3 but will only have effect when the interface is in a non-trunking mode. Switch1(config-if-range)#end Switch1#wr %SYS-5-CONFIG_I: Configured from console by console Building configuration... [OK] Switch1# Switch2 Switch#enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#int fa0/1 Switch(config-if)#switchport mode access Switch(config-if)#switchport access vlan 20 Switch(config-if)#spanning-tree portfast %Warning: portfast should only be enabled on ports connected to a single host. Connecting hubs, concentrators, switches, bridges, etc... to this interface when portfast is enabled, can cause temporary bridging loops. Use with CAUTION %Portfast has been configured on FastEthernet0/1 but will only have effect when the interface is in a non-trunking mode. Switch(config-if)#exit Switch(config)#int fa1/1 Switch(config-if)#switchport mode access Switch(config-if)#switchport access vlan 20 Switch(config-if)#spanning-tree portfast %Warning: portfast should only be enabled on ports connected to a single host. Connecting hubs, concentrators, switches, bridges, etc... to this interface when portfast is enabled, can cause temporary bridging loops. Use with CAUTION %Portfast has been configured on FastEthernet1/1 but will only have effect when the interface is in a non-trunking mode. Switch(config-if)#exit Switch(config)#int fa2/1 Switch(config-if)#switchport mode access Switch(config-if)#switchport access vlan 20 Switch(config-if)#spanning-tree portfast %Warning: portfast should only be enabled on ports connected to a single host. Connecting hubs, concentrators, switches, bridges, etc... to this interface when portfast is enabled, can cause temporary bridging loops. Use with CAUTION %Portfast has been configured on FastEthernet2/1 but will only have effect when the interface is in a non-trunking mode. Switch(config-if)#end Switch#wr %SYS-5-CONFIG_I: Configured from console by console Building configuration... [OK] Switch# Настроить trunk-порты между свитчами Switch4 Switch#enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#int gi0/2 Switch(config-if)#switchport mode trunk Switch(config-if)#switchport trunk allowed vlan 10,20,30 Switch(config-if)#end Switch#wr %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/2, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/2, changed state to up %SYS-5-CONFIG_I: Configured from console by console ' Translating "wr'"...domain server (255.255.255.255) Switch3 Switch#enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#int gi3/1 Switch(config-if)#switchport mode trunk Switch(config-if)#switchport trunk allowed vlan 10,20,30 Switch(config-if)#exit Switch(config)#int gi5/1 Switch(config-if)#switchport mode trunk Switch(config-if)#switchport trunk allowed vlan 10,20,30 Switch(config-if)#end Switch#wr %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet5/1, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet5/1, changed state to up %SYS-5-CONFIG_I: Configured from console by console Building configuration... [OK] Switch# Switch2 -> Switch3 Switch#enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#int gi5/1 Switch(config-if)#switchport mode trunk Switch(config-if)#switchport trunk allowed vlan 10,20,30 Switch(config-if)#end Switch#wr %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet5/1, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet5/1, changed state to up %SYS-5-CONFIG_I: Configured from console by console Building configuration... [OK] Switch# Switch# B1 IP interface Switch1#enable Switch1#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch1(config)#interface vlan 10 Switch1(config-if)#ip address 192.168.10.2 255.255.255.0 Switch1(config-if)#no shutdown Switch1(config-if)#end Switch1#wr %LINK-5-CHANGED: Interface Vlan10, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan10, changed state to up %SYS-5-CONFIG_I: Configured from console by console Building configuration... [OK] Switch1# Switch2 Switch#enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#interface vlan 10 Switch(config-if)#ip address 192.168.10.3 255.255.255.0 Switch(config-if)#no shutdown Switch(config-if)#end Switch#wr %LINK-5-CHANGED: Interface Vlan10, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan10, changed state to up %SYS-5-CONFIG_I: Configured from console by console Building configuration... [OK] Switch3 Switch#enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#interface vlan 10 Switch(config-if)#ip address 192.168.10.4 255.255.255.0 Switch(config-if)#no shutdown Switch(config-if)#end Switch#wr %LINK-5-CHANGED: Interface Vlan10, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan10, changed state to up %SYS-5-CONFIG_I: Configured from console by console Building configuration... [OK] Switch4 Switch#enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#interface vlan 10 Switch(config-if)#ip address 192.168.10.5 255.255.255.0 Switch(config-if)#no shutdown Switch(config-if)#end Switch#wr %LINK-5-CHANGED: Interface Vlan10, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan10, changed state to up %SYS-5-CONFIG_I: Configured from console by console Building configuration... [OK] C2 ping C:\>ping 192.168.10.12 Pinging 192.168.10.12 with 32 bytes of data: Reply from 192.168.10.12: bytes=32 time<1ms TTL=128 Reply from 192.168.10.12: bytes=32 time<1ms TTL=128 Reply from 192.168.10.12: bytes=32 time<1ms TTL=128 Reply from 192.168.10.12: bytes=32 time<1ms TTL=128 Ping statistics for 192.168.10.12: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 0ms, Maximum = 0ms, Average = 0ms C:\>ping 192.168.10.13 Pinging 192.168.10.13 with 32 bytes of data: Reply from 192.168.10.13: bytes=32 time<1ms TTL=128 Reply from 192.168.10.13: bytes=32 time<1ms TTL=128 Reply from 192.168.10.13: bytes=32 time<1ms TTL=128 Reply from 192.168.10.13: bytes=32 time<1ms TTL=128 Ping statistics for 192.168.10.13: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 0ms, Maximum = 0ms, Average = 0ms C:\> Pinging 192.168.20.12 with 32 bytes of data: Reply from 192.168.20.12: bytes=32 time<1ms TTL=128 Reply from 192.168.20.12: bytes=32 time<1ms TTL=128 Reply from 192.168.20.12: bytes=32 time<1ms TTL=128 Reply from 192.168.20.12: bytes=32 time<1ms TTL=128 Ping statistics for 192.168.20.12: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 0ms, Maximum = 0ms, Average = 0ms C:\>ping 192.168.20.13 Pinging 192.168.20.13 with 32 bytes of data: Reply from 192.168.20.13: bytes=32 time<1ms TTL=128 Reply from 192.168.20.13: bytes=32 time<1ms TTL=128 Reply from 192.168.20.13: bytes=32 time<1ms TTL=128 Reply from 192.168.20.13: bytes=32 time=1ms TTL=128 Ping statistics for 192.168.20.13: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 0ms, Maximum = 1ms, Average = 0ms Switch3 Switch>show interfaces trunk Port Mode Encapsulation Status Native vlan Gig3/1 on 802.1q trunking 1 Gig5/1 on 802.1q trunking 1 Port Vlans allowed on trunk Gig3/1 10,20,30 Gig5/1 10,20,30 Port Vlans allowed and active in management domain Gig3/1 10,20,30 Gig5/1 10,20,30 Port Vlans in spanning tree forwarding state and not pruned Gig3/1 10,20,30 Gig5/1 10,20,30 Switch>show vlan brief VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa0/1, Fa1/1, Fa2/1, Fa4/1 10 VLAN10_USERS active 20 VLAN20_USERS active 30 VLAN30_USERS active 1002 fddi-default active 1003 token-ring-default active 1004 fddinet-default active 1005 trnet-default active Switch> Switch2 Switch>show vlan brief VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa3/1, Gig4/1 10 VLAN10_USERS active 20 VLAN20_USERS active Fa0/1, Fa1/1, Fa2/1 30 VLAN30_USERS active 1002 fddi-default active 1003 token-ring-default active 1004 fddinet-default active 1005 trnet-default active Switch>show interfaces trunk Port Mode Encapsulation Status Native vlan Gig5/1 on 802.1q trunking 1 Port Vlans allowed on trunk Gig5/1 10,20,30 Port Vlans allowed and active in management domain Gig5/1 10,20,30 Port Vlans in spanning tree forwarding state and not pruned Gig5/1 10,20,30

## Вывод по дополнительному заданию

Конфигурация VLAN выполнена корректно: ПК внутри VLAN10 и VLAN20 успешно обмениваются ICMP (ping). Trunk порты подтверждены командами show interfaces trunk, а принадлежность портов VLAN подтверждена show vlan brief. Настройка SVI позволила задать IP адрес управления коммутаторами для дальнейшего администрирования.

# Общий вывод по лабораторной работе

Лабораторная работа помогла понять, как устроены интерфейсы маршрутизатора и коммутатора, как подбираются модули расширения и почему важно внимательно проверять интерфейсы при соединении устройств. Также закреплен навык консольного доступа и базовой диагностики через команды show. Дополнительное задание с VLAN показало, как на уровне коммутатора логически разделять одну физическую сеть на несколько сегментов и проверять корректность конфигурации.